# Hover Constrained Nonlinear Wrench Authority and Control Allocation for Twin Only and Tail Assisted VTOL Architectures

Cameron Thomas

Department of Aerospace Engineering

Simplified AIAA Style Technical Report

# Abstract

This report evaluates the instantaneous nonlinear actuator-to-wrench authority of twin-only and tail-assisted vectored-thrust VTOL architectures while maintaining hover. Each main propulsor has independently commanded thrust and two gimbal angles. The tail-assisted configuration additionally includes an aft propulsor. An exact nonlinear wrench model, bounded weighted least-squares allocator, scaled Jacobian analysis, and hover-constrained force/moment maps were used. Both architectures are locally rank six over the sampled gimbal grid. However, the twin-only vehicle has a weak pitch-control direction: its feasible pitch-moment range is approximately plus or minus 0.15 N m and its smallest scaled singular value near hover is approximately 0.02. The tail-assisted architecture expands the central pitch-moment envelope to approximately -2.6 to +1.6 N m and increases the smallest scaled singular value to approximately 0.25. The results demonstrate that rank alone is not a sufficient measure of practical controllability. The tail provides its main benefit through a direct longitudinal thrust-moment lever and improved constrained allocation, although a matched mass/inertia/thrust study is required to quantify the contribution independently of increased installed thrust.

# Nomenclature

| Symbol | Definition |
| --- | --- |
| alpha, beta | Main-propulsor fore/aft and lateral gimbal angles |
| delta | Tail-propulsor tilt angle |
| F_i | Thrust force vector of propulsor i |
| M_i | Moment vector of propulsor i about the CG |
| r_i | Propulsor position relative to the CG |
| W | Body wrench [F_x, F_y, F_z, M_x, M_y, M_z]^T |
| J | Nonlinear wrench Jacobian, dW/du |
| J_s | Dimensionless scaled wrench Jacobian |
| kappa | Reaction-torque arm, 0.0144 m |
| l_t | Tail longitudinal moment arm, 0.350 m |
| u_max | Largest normalized actuator utilization |

# I. Introduction

The closed-loop proof-of-concept simulations showed a pronounced difference between twin-only and tail-assisted pitch-disturbance recovery. This report examines the actuator geometry and constrained authority behind that dynamic behavior. The objective is to determine whether a requested force/moment vector is physically achievable after thrust and gimbal limits are applied, rather than only determining whether an unconstrained controller can command it.

The study is quasi-static and hover constrained. It includes nonlinear thrust-vector geometry, reaction torque, actuator position limits, and allocation residuals. It does not include motor/gimbal bandwidth, structural flexibility, aerodynamics, or 6-DOF trajectory dynamics; those effects must be included in the next validation step.

# II. Nonlinear Wrench and Allocation Formulation

## A. Propulsor Wrench Model

Body axes follow the aerospace convention: x_b forward, y_b right, and z_b down. For each main propulsor,

| n(alpha, beta) = [sin alpha, cos alpha sin beta, -cos alpha cos beta]^T; F_i = T_i n_i | (1) |
| --- | --- |

Reaction torque and propulsor moment are modeled as

| Q_i = -sigma_i kappa F_i; M_i = r_i x F_i + Q_i | (2) |
| --- | --- |

The exact nonlinear body wrench is

| W(u) = [sum(F_i), sum(M_i)]^T | (3) |
| --- | --- |

For the twin-only reference geometry, r_1 = [0, +d, 0]^T and r_2 = [0, -d, 0]^T, where d = 0.300 m. The resulting pitch moment is

| M_y = -kappa F_y1 + kappa F_y2 | (4) |
| --- | --- |

Thus, common alpha primarily generates F_x, common beta primarily generates F_y, and differential beta generates the weak reaction-torque pitch moment. The direct trade is F_y versus M_y, not a literal F_x versus M_y demand on a single gimbal coordinate.

For the tail-assisted vehicle, F_3 = T_3[-sin delta, 0, -cos delta]^T and r_3 = [-l_t, 0, 0]^T. The tail pitch contribution is

| M_y3 = -l_t T_3 cos delta | (5) |
| --- | --- |

At trim, delta = 0, so dM_y3/dT_3 = -l_t = -0.350 N m/N and dM_y3/delta = 0. Tail thrust modulation, rather than tail tilt, is the first-order pitch-control mechanism in this geometry.

## B. Constrained Allocation and Scaled Jacobian

For requested wrench W_des, the allocator minimizes a weighted wrench-tracking residual and a command-change penalty, subject to thrust and gimbal position limits. Moment components receive greater priority than translation when the entire wrench cannot be achieved. Each map point maintains the hover condition F_z approximately equal to -mg.

| minimize ¦¦S_W[W(u) - W_des]¦¦^2 + lambda_u ¦¦S_u(u - u_prev)¦¦^2 | (6) |
| --- | --- |

Because actuator coordinates mix newtons and radians, raw Jacobian conditioning is not meaningful. The dimensionless scaled Jacobian uses wrench scales of mg and mgL_ref and actuator scales equal to half of the physical actuator travel.

| J_s = diag(1/W_scale) J diag(u_half-range) | (7) |
| --- | --- |

A value of u_max = 1 indicates that at least one actuator is at a position bound.

# III. Results and Discussion

## A. Force and Moment Envelopes

Fig. 1 Achievable longitudinal force and pitch moment while maintaining hover.

Fig. 2 Achievable lateral force and pitch moment while maintaining hover.

At nominal twin-only trim and beta = plus or minus 35 degrees, the differential-beta model predicts approximately plus or minus 0.122 N m of pitch moment, equivalent to approximately 6.1 rad/s^2 for I_yy = 0.020 kg m^2. The maps show a similarly narrow twin-only pitch-moment envelope, approximately plus or minus 0.15 N m.

The tail-assisted map expands the central pitch-moment range to approximately -2.6 to +1.6 N m. The range is asymmetric because the tail propulsor trims near 4 N and has unequal thrust-increase and thrust-decrease reserve within its 0.5 to 10 N bounds. The F_y/M_y map demonstrates the direct twin-only trade: lateral-force demand consumes the same finite beta authority required for reaction-torque pitch control. The tail-assisted configuration preserves considerably more pitch moment throughout the lateral-force sweep.

The F_x/M_y maps show indirect rather than direct coupling. Main alpha provides the primary F_x channel, whereas beta provides the twin primary M_y channel. Their ability to coexist decreases when gimbal travel, thrust limits, and the vertical-force constraint are simultaneously active.

## B. Allocation Residual and Actuator Utilization

Fig. 3 Allocation residual and maximum normalized actuator utilization, u_max.

The twin-only maps contain a relatively small central region of accurately achievable hover-compatible F_x/M_y requests. Near the boundary, u_max reaches one and the scaled wrench residual increases. Saturation alone is not a numerical failure; it is a limit of physical authority. It becomes a flight-control problem when the required attitude moment cannot be generated while the controller also needs translation and vertical force.

The tail-assisted map covers a much broader pitch-moment request range. Its outer boundary also saturates, as expected for a finite actuator set, but the attitude-priority feasible region is farther from nominal hover. The two residual maps use different requested-wrench and color-bar ranges, so their colors should not be treated as a direct point-by-point numerical comparison.

## C. Local Controllability and Conditioning

Fig. 4 Hover-constrained angular authority and scaled-Jacobian conditioning across the sampled gimbal grid.

Fig. 5 Minimum scaled singular value across the grid and central scaled-Jacobian spectrum.

Both sampled grids remain rank six, which confirms local mathematical controllability of the instantaneous map. Rank alone is misleading, however. Near the central operating condition, the twin-only minimum scaled singular value is approximately 0.02, compared with approximately 0.25 for the tail-assisted configuration. The twin-only vehicle therefore has a much weaker normalized wrench direction and is more sensitive to saturation, modeling uncertainty, and command scaling.

The hover-constrained pitch angular acceleration rises from roughly 7 rad/s^2 in the twin-only map to roughly 47 rad/s^2 in the tail-assisted map. Roll and yaw authority change because the supplied tail model differs in mass, inertia, and geometry; those axes are not a mass-normalized architecture comparison.

The conditioning heat maps display log10[kappa(J_s)], even though their abbreviated titles state scaled condition number. The logarithmic labeling must be retained or corrected before publication because a conventional condition number cannot be below one.

# IV. Engineering Implications

The results qualify the original architecture hypothesis. The twin-only model is locally full rank but lacks robust pitch authority under its stated limits. Its direct F_y/M_y coupling and weak reaction-torque pitch path lead to saturation before the full attitude-priority wrench can be sustained.

The tail-assisted model does not eliminate saturation, but it creates a direct pitch-control lever and improves the weakest allocation direction by roughly an order of magnitude. This preserves more main-propulsor gimbal capacity for force generation. The benefit is not solely attributable to the tail: the two reference vehicles differ in mass, inertia, geometry, and total installed thrust. A fair ablation should retain identical mass properties and main-propulsor limits while adding only the tail pitch channel.

# V. Conclusions

The nonlinear model does not reject the twin-only vehicle on rank alone; it remains rank six over the sampled static grid. It does show that the twin-only vehicle has a narrow practical pitch-moment envelope and poor margin in its weakest scaled direction. The tail-assisted vehicle materially increases pitch authority and improves constrained allocation by using aft thrust as a moment arm.

The next validation gate is a full 6-DOF combined simulation using this exact nonlinear allocator. It should log F_x,cmd versus F_x,actual, M_y,cmd versus M_y,actual, individual actuator commands, and u_max(t). This will separate instantaneous allocation loss from motor/gimbal lag, rate limits, altitude coupling, and nonlinear attitude dynamics.

# References

1. matlab/vtolWrench.m, matlab/vtolWrenchJacobian.m, and matlab/vtolDefaultParams.m, Nonlinear VTOL Wrench Model, project source code.

2. matlab/vtolConstrainedAllocate.m and matlab/vtolFeasibleWrenchMap.m, Constrained Allocation and Hover-Constrained Authority Mapping, project source code.

3. matlab/run_vtol_wrench_analysis.m and results/nonlinear_wrench_analysis.mat, Nonlinear Wrench Analysis Results, project analysis artifacts.

4. Closed Loop Disturbance Recovery Comparison of Twin Only and Tail Assisted Vectored VTOL Configurations, project technical report.